# Mobile Devices — A+

Course: A+   |   Week of 2026-09-15

Name (last name and class year only): ___________________________________

This companion enhances the PGCC curriculum; it does not replace it. Canvas is the official record of your grade.

Students with a 504 plan or IEP: your accommodations apply through PGCC Disability Support Services.

## Objectives

• Explain laptop hardware components and their upgrade paths

• Compare mobile OS platforms and synchronization methods

• Identify common mobile device connection standards

## Key Terms

SO-DIMM

Small Outline DIMM — laptop RAM form factor, physically shorter than desktop DIMM.

M.2

Small form-factor internal slot supporting SATA or NVMe storage drives.

NVMe

Non-Volatile Memory Express — PCIe SSD protocol significantly faster than SATA.

USB-C

Universal reversible connector carrying data, power, and video; standard since iPhone 15+.

Lightning

Apple proprietary connector used on iPhones prior to the iPhone 15 lineup.

Li-ion

Lithium-ion — standard rechargeable battery in laptops and phones; best kept 20–80% charged.

SoC

System on Chip — integrates CPU, GPU, modem, and other components on a single die.

ARM

Reduced instruction set processor architecture optimized for power efficiency in mobile devices.

BLE

Bluetooth Low Energy — designed for IoT sensors and wearables needing long battery life.

NFC

Near Field Communication — ~4 cm range radio used for contactless payments.

Gorilla Glass

Chemically strengthened glass using ion-exchange process for scratch and drop resistance.

Capacitive

Touchscreen technology detecting the electrical conductivity of a fingertip.

## The Concept

WHAT MAKES A LAPTOP DIFFERENT FROM A DESKTOP

Laptops pack full computing capability into a portable chassis, which demands different components. RAM uses the SO-DIMM form factor — physically shorter than desktop DIMM — so it fits in tight spaces. Storage connects through M.2 slots that support either the older SATA protocol (up to 600 MB/s) or the faster NVMe/PCIe protocol (over 5,000 MB/s on current drives). The key notch on the M.2 slot determines which protocols are wired.

BATTERY AND DISPLAY

Laptop batteries use lithium-ion chemistry. The best practice is to keep charge between 20% and 80%, avoiding full charges or complete discharges that stress the cell chemistry and reduce the total lifespan. Display types vary: IPS panels provide accurate color and wide viewing angles; TN panels respond faster at lower cost; OLED delivers perfect blacks because each pixel emits its own light.

MOBILE DEVICE CONNECTORS

USB-C became the universal standard after the EU mandated it for phones sold in Europe and Apple adopted it for iPhone 15+. It carries data, power (up to 240W with USB PD), and video over one reversible cable. Older Apple devices use Lightning; legacy Android used Micro-USB. USB-A is the host port on chargers, not a phone connector.

MOBILE PROCESSORS AND OPERATING SYSTEMS

Mobile devices use ARM-based processors organized into a System on Chip (SoC) — CPU, GPU, modem, memory controller, and neural processing engine all on one die. ARM's RISC design executes instructions with fewer transistors per operation, delivering better battery life than x86. Android is an open-source Linux-based OS; iOS runs on Apple's proprietary Darwin kernel.

SHORT-RANGE WIRELESS STANDARDS

Bluetooth Classic handles high-bandwidth applications like stereo audio (A2DP profile) and file transfers. Bluetooth Low Energy (BLE) cuts power consumption dramatically by using short, infrequent bursts — ideal for fitness trackers and IoT sensors that run for months on a coin cell. NFC operates at 13.56 MHz with a maximum range of about 4 cm; its primary use is tap-to-pay transactions where proximity itself confirms intent.

## Worked Examples

Example 1:

A user wants to upgrade their laptop's storage. The current M.2 drive reads speeds of 550 MB/s. They ask if a new drive will be faster. Diagnosis: 550 MB/s indicates a SATA M.2 drive (SATA III cap). Check the slot key type — if B+M, a SATA drive is present. If the motherboard also supports PCIe on that slot, upgrading to an NVMe drive could yield 10× faster reads. Confirm in the laptop manual before purchasing.

Example 2:

A user's iPhone 15 will not charge with their old Lightning cable. Resolution: iPhone 15 switched to USB-C. The Lightning cable is incompatible. Replace with a USB-C cable. Note: any USB-C cable will charge; for full data speed, use USB 3.x rated cable.

## Common Mistakes to Avoid

• Assuming any M.2 drive will work in any M.2 slot — check key type and supported protocols.

• Charging a Li-ion battery to 100% daily thinking it improves battery health — it accelerates degradation.

• Confusing Bluetooth Classic with BLE — they serve different use cases and are not interchangeable.

• Using a non-conductive stylus on a capacitive touchscreen — it will not respond; use a conductive stylus or finger.

• Confusing the SSID with the Wi-Fi password — SSID is the network name; the PSK is the password.

## My Notes

Use this space to write notes during class. Focus on what your facilitator emphasizes.

## Self-Check Questions

Cover the answers below. Try each question before reading the answer.

1. Which RAM form factor is used in laptops?

• A. SO-DIMM

• B. DIMM

• C. RDIMM

• D. UDIMM

2. What does keeping a Li-ion battery between 20–80% accomplish?

• A. Maximizes charge cycles by avoiding voltage extremes

• B. Charges the battery faster

• C. Prevents water damage

• D. Enables fast charging mode

3. Which connector became standard on iPhone 15+ and Android phones?

• A. USB-C

• B. Lightning

• C. Micro-USB

• D. USB-A

4. BLE is preferred over Bluetooth Classic for fitness trackers because:

• A. BLE uses far less power for low-data-rate sensors

• B. BLE streams higher quality audio

• C. BLE has longer range than Classic

• D. BLE is required for contactless payments

5. NFC is commonly used for:

• A. Contactless payments at ~4 cm range

• B. Long-range file transfer

• C. Internet connectivity

• D. GPS navigation

## Self-Check Answers

1. A — SO-DIMM is the laptop form factor — physically smaller than desktop DIMM.

2. A — Avoiding voltage extremes reduces chemical stress and extends total battery lifespan.

3. A — USB-C is the current universal standard for smartphones.

4. A — BLE's infrequent burst transmission model uses a fraction of the power of Bluetooth Classic.

5. A — NFC's ~4 cm range makes it ideal for tap-to-pay; proximity is the security factor.